Department of Electronics, University of York

Music Technology Year 2 – Voice Production and Synthesis (2015 – 2016)

Laboratory sessions: Voice analysis and synthesis

There are nine two-hour laboratory sessions timetabled for this module. The sessions take place in the PC Teaching Laboratory on the first floor of the Department of Electronics. This document is divided into seven labs to give you an indication of your expected progress throughout the term. Feel free to work through the document ahead of the labelled sessions. If you complete the lab work before the end of term you can use the remaining labs to begin work on your assignment.

Help and guidance is available throughout the laboratory sessions - please do ask as appropriate.

You can work as an individual or in pairs during the lab sessions, but it is important that you hand in a copy of your lab script and make clear who you have worked with (using their exam number not their name).

Introduction

During these laboratory sessions you will be analysing your own voice and some of your results will be used for synthesis later. These lab exercises are based on the material covered in the lectures, you will need to refer to your lecture notes throughout. You are expected to fill in your results in this Word document. It is important that you fill in the whole lab script in detail as it covers all the knowledge and skill you will need to complete the assignment.

The submission pages at the end of this script should be completed and submitted with the rest of the assignment and comprises 15% of the entire assessment for this module. Please ensure that units are indicated appropriately whenever measured values are recorded.

You will record various spoken items in the PC lab using a basic electrets microphone, and these recordings will be used for analysis during these lab sessions. It is important that these recorded sounds are of the highest quality you can achieve and therefore you are encouraged to think about how best to achieve appropriate recordings.

A note on file integrity (** IMPORTANT **)

You are responsible for preserving the integrity of your master data and maintaining appropriate back-ups.

Never edit the original wav or Word file. Always work on a copy and organise at least one back-up of the original. Do not assume that data left on a multi-user machine such as those in our PC lab will still be there the next time you log on. One key reason for making backups, apart from a possible hardware failure, is that if you cut a portion out of a wav file and then save that file, you will have lost the cut section from the original; this is very easy to do in an editing session!

You will need to use the IPA fonts, which should be available from the symbols in Lucida Sans Unicode in Word. If not see appendix II for how to download IPA fonts to your machine.

If working in pairs ensure you both have copies to work from in case one of you is absent one week.

Recording and preparing your data

In order to analyse speech sounds you will be recording your own speech using GoldWave (WWW-1) or Audacity, which can be found under the Start menu. Some useful GoldWave controls are shown in figure 1.

Save a master and a copy of the files you create. You will use these files throughout the lab sessions.

Exam number:

Partner’s exam number (if applicable):

Recorded voice type:  Male / Female

Lab 1

1. 	Record the phrase “The Department of Electronics at the University of York” at a slow and deliberate speech rate. Listen critically to your recording and redo if appropriate (see Appendix I for details of waveform selection, playing and trimming in Goldwave).  If you are having trouble recording, download the example voice recording from the module webpage.

1.1 	Filename for .wav of phrase:

Lab1 – Slow.wav

1.2 	Note any thoughts (below) about the quality of the recording and what (if any) actions you took to improve the overall recording quality:

Recorded using condenser mic with pop shield and reflection filter. Quality is good but there’s a lot of sibilant sounds/salival sounds.

1.3 	Listen carefully to your recording and write an IPA (Wells, 1989 and Appendix II) transcription of the phrase as you pronounced it:

The Department of Electronics at the University of York

ðʌ dʌpɑ:tmɛnt ɒv ɛlɛktɹɒnɪks æt ðʌ u:ni:vɜ:sɪti: ɒv jʊək

1.4 	Measure the total phrase length (normal speech rate):

8.740s

2. 	Select five vowels (monophthongs) from your transcription (above) of the phrase “The Department of Electronics at the University of York”.

2.1 	Measure the length of each of these five vowels using GoldWave or Audacity and fill in the table below.

| Five vowels from “The Department of Electronics at the University of York” (normal speech rate) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vowels | depart | artment | of | elec | tronics |
| IPA | ɑ: | ɛ | ɒ | ɛ | ɪ |
| duration | 0.197s | 0.074s | 0.146s | 0.104s | 0.087s |

2.2 	Comment on any issues determining the vowel lengths:

Hard to find where sounds end and others begin – they seem to meld into each-other

3. 	Select five consonants from your transcription (above) of the phrase “The Department of Electronics at the University of York”.

3.1 	Measure the duration of each of these five consonants using GoldWave or Audacity and fill in the table below.

| Five consonants from “The Department of Electronics at the University of York” (normal speech rate) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Consonants (in context) | the | departm | departme | artment | of |
| IPA | ð | d | p | t | v |
| duration | 0.046s | 0.038s | 0.072 | 0.078s | 0.087s |

3.2 	Comment on any issues determining the consonant lengths and any noticeable differences between vowel and consonant lengths:

It’s even harder to unpick the blend between the two sounds. They are much shorter and tend to incorporate parts of the vowel as it changes.

4. 	Record the phrase “The Department of Electronics at the University of York” again, but this time at a fast speech rate and save to a wav file with an appropriate name. (Again, save a master and a copy for editing).

4.1 	Filename for .wav of phrase fast speech rate:

Lab1 – Fast.3.wav

4.2 	Measure the total phrase length (fast speech rate):

3.366s

4.3 	Calculate the normal:fast speech rate length ratio:

1:0.385

4.4 	Measure the length of the same vowels and consonants used in Lab 1 using GoldWave or Audacity also calculating the normal:fast rate ratios for each vowel and consonant and fill in the tables below.

| Five vowels from “The Department of Electronics at the University of York” (fast speech rate) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vowels (in context) | depart | artment | of | elec | tronics |
| IPA | ɑ: | ɛ | ɒ | ɛ | ɪ |
| duration | 0.126 | 0.034 | 0.034 | 0.026 | 0.048 |
| rate ratio | .640 | .459 | .233 | .25 | .55 |

| Five consonants from “The Department of Electronics at the University of York” (fast speech rate) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Consonants | the | departm | departme | artment | of |
| IPA | ð | d | p | t | v |
| duration | 0.01 | 0.021 | 0.03 | 0.035 | 0.049 |
| rate ratio | .217 | .55 | .417 | .449 | .56 |

4.5	Comment on any differences in lengths and/or normal:fast rate ratios between the two versions of the phrase:

The sounds are shorter. It’s even harder to hear where the transition is. The variation is not consistent between all sounds.

Lab 2

5. 	Using GoldWave, record the following vowels on a steady pitch: /i/; /ɛ/; /æ/; /ɑ/;  /ɒ/; /u/ (The IPA is listed in Appendix II for convenience).

5.1 	Store the recordings as individual .wav files using intuitively descriptive filenames recorded in the table below:

| vowel | filename |
| --- | --- |
| /i/ | Lab2 - /i/.wav |
| /ɛ/ | Lab2 - /ɛ/.wav |
| /æ/ | Lab2 - /æ/.wav |
| /ɑ:/ | Lab2 - /ɑ//.wav |
| /ɒ/ | Lab2 - /ɒ/.wav |
| /u:/ | Lab2 - /u//.wav |

6. 	Open the voice analysis program Praat (WWW-2). In the Praat objects window (on the left hand side) select and open one of the vowel wav files (‘read from file’).

Select the View & Edit button and the file is opened in a new Edit window as a waveform and a spectrogram.

Explore the viewing options of praat. E.g. zooming options using the view button, play using view or spacebar.

Note that the f0 value for the mouse selection is shown on the right hand side where the frequency scale is magnified for f0 measurements, the value on the left hand side should NOT be used here as it is for the spectrogram frequency axis.

6. 1 	Measure the duration of the all vowels and record them in the table below

To do this select the vowel carefully (mouse click and hold at the left hand end and drag to the right hand end and release the mouse button) and note the value in seconds that is shown under the display.

6.2 	Measure the approximate fundamental frequency (f0) of all the vowels and record them in the table below.

To do this select the steady state of the vowel then going to Pitch – Get Pitch. (check that Pitch-Show pitch is selected)

| vowel | duration | fundamental frequency (f0) |
| --- | --- | --- |
| /i/ | 0.265s | 181Hz |
| /ɛ/ | 0.239s | 174Hz |
| /æ/ | 0.238s | 161Hz |
| /ɑ:/ | 0.441s | 158Hz |
| /ɒ/ | 0.234s | 155Hz |
| /u:/ | 0.407s | 159Hz |

6.3 	Comment on the consistency of your duration and F0 values:

Mostly they seem to be listed in descending pitch, the exception being u: – although I imagine pronounced in a more RP way it would be lower (I pronounce it more nasally). The duration are consistent other than a: and u: which are longer. This makes a bit of sense as they tend to be vowels that I would stretch out in words.

7. 	Use GoldWave or Audacity to record and save a wav file of any three vowels from /i/; /ɛ/; /æ/; /ɑ:/;  /ɒ/; /u:/ spoken with a  falling pitch.

7.1 	Measure the maximum and minimum f0 values and estimate the rate of f0 change of each vowel, filling in the table below.

| vowel | minimum f0 | maximum f0 | f0 rate of change |
| --- | --- | --- | --- |
| /i/ | 130.1Hz at 3.538526s | 186.7Hz at 2.89582s | 56.6/.642706=88Hz/s |
| /ɛ/ | 130.9Hz at 3.597914s | 187.5Hz at 2.700367s | 56.6/.897547=63Hz/s |
| /æ/ | 126.2Hz at 3.354150s | 175.7Hz at 2.393657s | 49.5/.960493=52Hz/s |

7.2 	Export the f0 data for each vowel into Excel or Matlab and plot the contours below with the maximum and minimum clearly labelled.

Contour Plot 1 Vowel (IPA): /i/

Contour plot 2

Vowel (IPA):/æ/

Contour plot 3  Vowel (IPA): ɛ

Lab 3

8. 	Repeat the recordings and measurements from 7. but with a rising f0 (three vowels).

8.1	Fill in the table below with the measurements and plot the contours as in section 7 in the spaces below.

| vowel | minimum f0 | maximum f0 | maximum f0 rate of change |
| --- | --- | --- | --- |

Contour plot 1

Vowel (IPA):

Contour plot 2

Vowel (IPA):

Contour plot 3

Vowel (IPA):

8.2 	Comment on any similarities or differences between the falling and rising f0 patterns studied in 7 and 8.

Vowel (monophthong) analyses

9. 	Use Praat to find the first two formants (F1 and F2) for each of your six vowels spoken on a steady pitch and fill in the table below.

This is done similar to the f0 exercise in the previous lab using the formant tab

| vowel | F1 | F2 |  |  |
| --- | --- | --- | --- | --- |
| /i/ | 545.3388150616257 | 2501.1862569075456 |  |  |
| /ɛ/ | 1004.8520549850004 | 2304.691972628361 |  |  |
| /æ/ | 1267.7899724397907 | 2312.9188245434952 |  |  |
| /ɑ:/ | 865.3055760389218 | 1303.5309955802165 |  |  |
| /ɒ/ | 756.7045553628125 | 1575.4090008686253 |  |  |
| /u:/ | 329.6131188302091 | 1988.088766425628 Hz |  |  |

9.1 	Use Excel or Matlab to plot the F1 against F2 values for all six vowels on axes organised as shown in figure 2. Note that the data plots on the graph should represent each vowel and if using excel the axes need to be aligned as shown above using ‘format axis’

Plot showing the first and second formant values of six vowels:

9.2 	Comment on the F1 against F2 plot for your six vowels with respect to the articulation of the vowels and the expected results compared to your findings based on your recordings:

Lab 4

Vowel (diphthong) analyses

10. 	Select a word that includes the diphthong /aɪ/ and another that includes /ɪə/.

Record your selected two words with one on a rising and one on a falling pitch using GoldWave (or Praat – something to explore).

10.1 	Save each as a wav file with an appropriate name. Fill in the table below.

| Word | Diphthong (IPA) | pitch (rising/falling?) | filename |
| --- | --- | --- | --- |

10.2 	Use Praat to find the duration and maximum and minimum f0 values for each diphthong and record them in the table below.

| diphthong | duration | minimum f0 | maximum f0 |
| --- | --- | --- | --- |

10.3	Make appropriate use of Praat to enable you to clearly plot the first and second formant transitions during the two diphthongs below:

You can use Excel, Matlab or Praat pictures / screen shots as appropriate.

Plot of formant transitions in diphthong:

Plot of formant transitions in diphthong:

10.4 	Measure the extreme values of F1 and F2 during the transitions. Estimate the rate of change of each F1 and F2 during the transitions in each diphthong and record in the tables below.

| diphthong | minimum F1 | maximum F1 | minimum F2 | maximum F2 |
| --- | --- | --- | --- | --- |

| diphthong | rate of change in F1 | rate of change in F2 |
| --- | --- | --- |

10.5 	Comment on your findings:

Lab 5

Consonant analyses

11. 	Select a word that includes a voiceless fricative and another that includes its voiced counterpart (place and manner of each is the same).

11.1 	Record these two words using GoldWave (or Praat – something to explore to speed your task).

11.2 	Use Praat to find the durations of the two fricatives.

| File name | fricative | Word | duration |
| --- | --- | --- | --- |
| Zip.wav | Voiced alveolar fricative | /zɪp/ |  |
| Sip.wav | Voiceless alveolar fricative | /Sɪp/ |  |

11.3 	By observing the spectrogram of each, note their main acoustic properties, and in particular, indicate how voicing manifests itself on a spectrogram for the two sounds chosen:

12. 	Record the following words: /bɪt/, /pɪt/, /dɪp/, /tɪp/, /gæp/, /kæp/ and save to separate wav files.

12.1 	Measure the burst centre frequencies and voice onset time for each initial consonant.

| File name | word | burst centre frequency | voice onset time |
| --- | --- | --- | --- |
|  | /bɪt/ |  |  |
|  | /pɪt/ |  |  |
|  | /dɪp/ |  |  |
|  | /tɪp/ |  |  |
|  | /gæp/ |  |  |
|  | /kæp/ |  |  |

12.2 	Comment on the acoustic properties of the plosives you have analysed:

Lab 6

Creation of a speech synthesis system

13. 	Using PD (PureData - WWW-3), in the remainder of these labs you will create a voice synthesis system that includes three formants with a voiced and voiceless sound source.

13.1	List the main components needed for your PD synthesis:

Voice source

Filters for formants F1 F2 F3 etc

13.2 	List the main user interface elements:

Monophthong synthesis

14. 	Synthesize with a constant f0 copies of three of the monophthong vowels you analysed in section 2. and create wav files based on the predicted formant values for those vowels (Howard and Angus, 2008).

| vowel | filename |
| --- | --- |

14.1 	Use Praat to establish how closely the first three formants of your synthesized monophthongs match those of the originals you recorded and analysed.

Comment on the match between synthetic and natural vowels:

15.  	Based on your observations and comments in 14.1, modify your synthesis to achieve a closer formant match to your natural original monophthongs.

15.1 	Comment on the changes made to achieve a closer match between synthetic and natural vowels:

16. 	Create a more natural version of these three synthesized monophthongs by setting an appropriate f0 contour.

16.1 	Indicate the nature of the f0 contour used and the impact of using this contour:

Lab 7

Diphthong synthesis

17.  	Synthesize copies of the two diphthongs recorded in section 10 (without any consonants that might be in your chosen words but with a natural f0 contour).

| diphthong | filename |
| --- | --- |

17.1 	Use Praat to establish how closely your synthesized diphthongs match the originals.

Comment on match between synthetic and natural diphthongs:

17.2 	Based on your observations, modify your synthesis to achieve a closer match to your natural original diphthongs.

Comment on the changes made to achieve a closer match between synthetic and natural vowels:

References

Howard, D.M., and Murphy, D.T. (2008). Voice Science Acoustics and Recording, San Diego: Plural Press.

Howard, D,M., and Angus,J. (2009). Acoustics and Psychoacoustics 4th Ed., US: Taylor & Fracis

Wells, J.C. (1989). Computer coded phonemic notation of individual languages of the European Community, (IPA transcription), Journal of the International Phonetic Association, 19, 32-54.

WWW-1: http://www.goldwave.com/ (last accessed: 03.01.2014).

WWW-2: http://www.fon.hum.uva.nl/praat/ (last accessed: 03.01.2014).

WWW-3: http://puredata.info/ (last accessed: 03.01.2014). 
Appendix I – Waveform selection, trimming and playing in GoldWave

Basic actions on a recorded waveform include selection of a section, trimming and playing.

Figure ApI-1: Some useful GoldWave toolbar buttons.

Selection can be carried out in one of two ways depending on your preference: (1) using the left and right mouse buttons to select the left and right hand side of a section of the waveform respectively, or (2) using a mouse click and hold at the left hand end followed by a mouse sweep to the right hand end and button release to select the portion of interest. This choice is made under Options-Window and checking Use left and right mouse buttons... for option (1) and un-checking it for option (2).

Trimming the waveform can be carried out in one of two ways having made your selection depending on whether you want the original to remain intact with the trimmed section in a new file or not. You can either “trim” the file itself (Control-T or Edit-Trim or the Trim button shown in figure ApI-1), or create a new file with appropriate parameters (Control-N or File-New or the new button shown in figure ApI-1), copy the section (Control-C or Edit-Copy or the Copy button shown in figure ApI-1) you want from your file and paste it into the new file (Control-V or Edit-Paste or the Paste button shown in figure ApI-1) and then trim it (Control-T or Edit-Trim or the Trim button shown in figure ApI-1) to remove the data space that comes with the new file. Having created your new file, it is normal practice to amplitude ramp the start and end if appropriate to avoid clicks (select the waveform portion to ramp in or out and use Effect-Volume-Fade In/Fade Out or the Fade in or Fade out  buttons shown in figure ApI-1), and then save it (File-save or save button shown in figure ApI-1) with an appropriate name.

When making your selections, particularly when selecting from a section of running speech or singing, do listen carefully to the sound you are selecting to ensure that it is complete and that it includes no part of the surrounding sounds.

Playing the complete waveform is achieved by pressing the spacebar, and the selection in played using Control-spacebar. There are three play buttons on the transport control (see figure 1) and these can be configured to play parts of the waveform by pressing the transport control button (see figure 1) and setting each appropriately.

Help is available under the Help button (see figure ApI-1) and the function of other toolbar buttons can be found by placing the mouse cursor over the button of interest and waiting a moment.

Appendix II – IPA (International Phonetic Alphabet)

Below is an IPA table of example words to help you with your transcription. Note that the examples are based on Received Pronunciation (RP) and do not necessarily refer to the same word if spoken with a different accent

| Vowels |  |  | Diphthongs |  |
| --- | --- | --- | --- | --- |
| RP | Example |  | RP | Example |
| ɑ: | Father |  | eɪ | Day, pain |
| I | Sit |  | aɪ | My, wise |
| i: | See |  | ɔɪ | Boy |
| ɛ | Bed |  | əʊ | No, tow |
| ɜ: | Bird |  | aʊ | Now |
| æ | Lad, cat, ran |  | ɪə | Near, here |
| ʌ | Run, enough |  | ɛə | Hair, there |
| ɒ | Not, wasp |  | ʊə | Tour |
| ɔ: | Law, caught |  | ju: | pupil |
| ʊ | Put, wood |  |  |  |
| u: | Soon, through |  |  |  |
| ə | About, winner |  |  |  |
| English Consonants |  |  |  |  |
|  |  | Examples |  |  |
| k |  | Cat, kill, skin, queen, thick |  |  |
| g |  | Go, get, beg |  |  |
| ɲ |  | Singer, ring |  |  |
| tʃ |  | Chair, nature, teach |  |  |
| dʒ |  | Gin, joy, edge |  |  |
| t |  | Two, sting, bet |  |  |
| d |  | Do, odd |  |  |
| n |  | No, tin’ |  |  |
| p |  | Pen, spin, tip |  |  |
| b |  | But, web |  |  |
| m |  | Man, ham |  |  |
| j |  | Yes |  |  |
| ɹ |  | run |  |  |
| l |  | Left, bell |  |  |
| w |  | We |  |  |
| ʍ |  | What |  |  |
| ʃ |  | She, sure, emotion, leash |  |  |
| ʒ |  | Pleasure, beige |  |  |
| S |  | See, city, pass |  |  |
| z |  | Zoo, rose |  |  |

Tables adapted from http://www.ling.upenn.edu/courses/ling001/English.html (Last accessed 10/ 1/2014) and http://simple.wikipedia.org/wiki/IPA_chart_for_English (Last accessed 10/ 1/2014)

The IPA characters needed for this lab should be available through symbols in Word in the Lucida Sans Unicode font.

If you would like to download a font to make all IPA symbols accessible the relevant links can be found below:

To Download the IPA font follow the link below

http://scripts.sil.org/cms/scripts/page.php?item_id=CharisSIL_download (last accessed 10/01/2014)

You will also need to download an IPA keyboard. Note there are different downloads for PC and Mac.

http://scripts.sil.org/cms/scripts/page.php?item_id=UniIPAKeyboard (last accessed 10/02/2014)

If using a MAC the following instructions may be of use:

http://linguisticmystic.com/2007/03/08/using-ipa-fonts-with-mac-os-x-the-comprehensive-guide/ (Last Accessed 10/01/2014)

Voice Production and Synthesis

Lab Submission

The following pages should be filled in from your completed lab script, and submitted alongside the rest of your assignment for the Voice Synthesis module. You do not need to submit the rest of the lab script.

Exam number:

Partner’s exam number (if applicable):

Recorded voice type:  Male / Female

- Insert your IPA transcription from 1.3 here:

- Comment on the transcription of your recording compared to your expected IPA of the phrase:

- Insert one of your three plots from 7.2 here:

- Insert your table from 9 here:

| - vowel | F1 | F2 |  |  |
| --- | --- | --- | --- | --- |
| /i/ |  |  |  |  |
| /ɛ/ |  |  |  |  |
| /æ/ |  |  |  |  |
| /ɑ:/ |  |  |  |  |
| /ɒ/ |  |  |  |  |
| /u:/ |  |  |  |  |

- Insert your plot from 9.1 here:

Plot showing the first and second formant values of six vowels:

- Insert your two plots from 10.3 here stating the diphthongs using IPA: